grabmode, a text/XWindows mode grabber

======================================

Another tool to get SVGAtextMode up and running. While the clock probe just

tells you what pixel clock is actually in use, this program grabs the entire

text mode line from the VGA card, and shows it on the screen in a format

compatible with the TextConfig file.

Syntax:

./grabmode: version 0.5.0

Usage: ./grabmode [options]

Options: -n  Don't program VGA hardware

-d  print debugging information

-X  Output compatible with XF86Config mode line

-T  Output compatible with TextConfig mode line

-h  print usage information

Grabmode can give you the mode line in two formats: one compatible with

SVGAtextMode, and one compatible with XF86Config.

Normally grabmode is used WITHOUT the -T or -X argument. It finds out from

the VGA hardware whether you are running it from text mode or graphics mode,

and adjusts its output accordingly.

When executed from the same text mode, you get the following results

(example):

./grabmode -T  (or just "grabmode", since this IS text mode)

./grabmode: Please be patient. This may take a while (up to 1 minute)

"132x64"   75.165   1056 1088 1240 1336   768 775 776 800   +Hsync +Vsync  font 8x12   # 56.262kHz/70.33Hz

./grabmode -X

./grabmode: Please be patient. This may take a while (up to 1 minute)

"1056x768"   75.167   1056 1088 1240 1336   768 775 776 800   +Hsync +Vsync  # 56.263kHz/70.33Hz

A word of warning: If you use "grabmode -T" from a graphical screen (like

from X-Windows) it will give rather strange results. The example below was

executed from a 1024x768 X-windows screen:

"128x768"   75.165   1024 1064 1216 1336   768 775 776 800   +Hsync +Vsync  font 8x1   # 56.261kHz/70.33Hz

While "grabmode -X" would read (correctly):

"1024x768"   75.165   1024 1064 1216 1336   768 775 776 800   +Hsync +Vsync  # 56.261kHz/70.33Hz

Since graphical screens have nothing to do with hardware assisted font

rendering (as from VGA text mode - let's forget about accelerators for a

while), they set the "character height" to 1 pixel (in fact they have to,

otherwise each line gets repeated 16 times, if they leave the "16" from the

9x16 character mode in there!).

If you still want to do this, e.g. in order to grab the X-Windows timings to

use them in TextConfig, you'll have to adapt the width and height of the

extracted mode, and add an appropriate "font ..." line to it. grabmode

cannot do this conversion for you, since it doesn't know what font size you

want.

You can use this tool either to check the current mode (if something went

wrong, you want to know what REALLY got programmed in that slap of silicon

they call the "VGA chip"), or to grab an existing text mode and import it

into SVGAtextMode.

And to enable you grab the text modes from DOS, Kenneth Albanowski has

prepared a DOS port of grabmode, "grabmode.exe". A clever move! This way you

can get the modes from DOS, if the Linux boot doesn't recognize your VGA

card.

Note that the options for this DOS version must be entered with a "/"

prepended, and not a "-" as in UNIX.

Enjoy!